[image: image1.png]


[image: image2.jpg]


[image: image3.jpg]l




[image: image4.jpg]



Installing Your Pullout Shelf

Installing your new pullout shelf should take just a few minutes, a few tools and no experience for most applications.  Let’s start with identifying what type of cabinets you have.  We must determine if we have “face frame” cabinets or “frameless”.  Take a look below and decide which you have.

The cabinet on the left is a frameless cabinet.


The cabinet on the right is a face frame cabinet.  


Frameless Installation

When installing the pullouts in the frameless cabinet, it should just be a matter of attaching the bottom mount to the pullout/drawer and the slide to the inside wall of the cabinet.

1. Make sure your pullout slides will not interfere with your hinges

2. Mount epoxy slides to bottom of pullout/drawer, front end flush to the front of the pullout/drawer 

3. Mark on inside cabinet wall screw locations (Min 4 per side) at appropriate height and level from front to back.  Use the long slotted openings first to allow for adjustment, then once all adjustments are made secure with screws through round holes 

4. Pre-drill screw holes at each mark, attach slide with screws. Leave some room for adjustment until “tuned”

5. Slide shelf/drawer into mounted slides and check for smoothness and level.

6. Done!

In our picture example you can install two pullouts here by placing the upper pullout slide on the shelf, leveling the slide to the front and attach with screws once again to the inside wall of the cabinet.  If there is no shelf, then repeat process for lower installation.  A frameless cabinet generally will restrict you to one pullout (side by side) per opening.  If you have an opening over 24” and want more than one pullout (side by side), you may have to modify your cabinet or buy special mounting hardware.  Contact me if you wish to explore this possibility.

Face Frame Cabinet Installation
Installing pullouts in the face frame cabinet can also be quick and easy.  If there is NO door intrusion as noted in your measurements, then we can simply attach the slides and be done.

1. Follow steps 1 & 2 from frameless instructions

2. Attach socket to rear of slide by pushing it on.  It is adjustable so don’t worry about how far you push it on.

3. Screw front of drawer slide to inside of face frame leaving approx 3/32” from front edge of slide to front of face frame.

4. Level the slide and while doing so, push the “socket” back until it is making contact with the rear inside of the cabinet.  Verify that the right/left measurement from the inside wall is the same from front to back on the slide.  Once this measurement is correct and we are level, put two screws in the socket and make sure that everything is level and square from front to back.

5. Repeat process for the other side.

A second unit may be installed above the bottom even with a shelf by repeating the installation process and using the shelf as a “locator” for the upper pullout.  If there is no shelf, then you simply repeat the installation process without the “locator” and just set the height to whatever you desire.

If there IS door intrusion, then some additional work will be required to accomplish the installation on the side where the intrusion is.   

1. Measure the distance from the inside of the door opening to the inside wall of the cabinet where the intrusion is (should be about 1”)

2. Add the amount of (I)ntrusion to the measurement from step 1

3. This amount is how much we will need to adjust our side mounting to allow for the intrusion and the face frame.

4. In most cases it will be easiest to have two cleats of wood built to the step 2 combined measurement attached vertically to the inside of the cabinet in two locations (one near the front and the other near the rear).  These can be glued and screwed to the inside of the cabinet and provide a most secure method of attachment.  Once these cleats are installed, the slide will be screwed directly to them for lower and upper pullout installations.

5. Use normal installation method for the other slide where this is NO intrusion.

These Instructions are not all-inclusive and will not cover all situations.  This will accomplish installation in the majority of cases.  

 By The Cabinet Guy





P.O. Box 5350    Port Arthur, Texas 77640


Phone: 1-866-937-7429    Fax:  1-866-642-9971


Email:  CabinetGuy@2kbees.com










  © The Cabinet Guy 2007


